AEROBOT RESEARCH AND INNOVATIVE ENGINEERING SOLUTIONS LLP (ARIES)

VGP Queens Town, 48/49, Chettipedu, Kancheepuram, Sriperumbudur, Tamil Nadu 602105, India

GSTIN 33ACEFA6282G1ZJ   |   sakthi@aerobotlabs.com

BILL OF MATERIALS

| PROJECT / ASSEMBLY Project Name: MIT UAV Product: Quadcopter Revision R0 Prepared By: ARIES | BOM DETAILS BOM No.: BOM-2526-MIT-UAV Date: 21 / 07 / 2026 Prepared For : MIT Status : FINAL |
| --- | --- |

| # | Component & Description | Qty | Unit Cost | Total Cost | Notes |
| --- | --- | --- | --- | --- | --- |
| A. Hardware Components |  |  |  |  |  |
| 1 | Frame Tarot X4 Quadcopter Frame (TL4X001) | 1 | ₹ 18,543.00 | ₹ 18,543.00 | 960 mm wheelbase; retractable landing gear and integrated PCB |
| 2 | Flight Controller & ESC Stack MicoAir743 V2 AM32 70A 30×30 Stack | 1 | ₹ 12,000.00 | ₹ 12,000.00 | FC STM32H743; ESC 70A rated for 2–6S |
| 3 | Motor Tarot TL96020 5008 340KV | 4 | ₹ 5,549.54 | ₹ 22,198.16 | 700 W peak power; 40 A max current |
| 4 | Propeller 18" Counter-Rotating Carbon Fiber 1855 (CW+CCW) | 3 | ₹ 1,936.38 | ₹ 5,809.14 | Matched to selected motor and frame |
| 5 | Battery 6S 8000 mAh LiPo | 3 | ₹ 12,277.00 | ₹ 36,831.00 | 15C–25C discharge rate recommended |
| 6 | Receiver GEPRC ELRS 915/2.4G Gemini Xrossband | 1 | ₹ 3,699.00 | ₹ 3,699.00 | PWM / CRSF output compatible with FC |
| 7 | Transmitter Radiomaster Boxer 2.4GHz (M2) – ELRS | 1 | ₹ 16,000.00 | ₹ 16,000.00 | ELRS radio controller |
| 8 | GPS & Compass CUAV NEO 3 Pro ¦ GNSS U-BLOX M9N CAN BUS | 1 | ₹ 17,000.00 | ₹ 17,000.00 | For position hold and autonomous flight |
| 9 | Battery Charger ToolkitRC M7 200W 10A 2–6S DC Smart Charger | 1 | ₹ 4,500.00 | ₹ 4,500.00 | Rated for 8000 mAh battery |
| Components Subtotal |  |  |  | ₹ 1,36,580.30 |  |
| B. Assembly, Testing & Consumables |  |  |  |  |  |
| 10 | Build | 8 | ₹ 500.00 | ₹ 4,000.00 | 8 hrs @ ₹500/hr |
| 11 | Integration | 3 | ₹ 500.00 | ₹ 1,500.00 | 3 hrs @ ₹500/hr |
| 12 | Bench testing | 5 | ₹ 500.00 | ₹ 2,500.00 | 5 hrs @ ₹500/hr |
| 13 | Flight test | 16 | ₹ 1,000.00 | ₹ 16,000.00 | 16 hrs @ ₹1,000/hr |
| 14 | Contingency | 5 | ₹ 500.00 | ₹ 2,500.00 | Buffer allowance |
| 15 | Consumables | 1 | ₹ 2,500.00 | ₹ 2,500.00 | Wiring, connectors, adhesives, etc. |
| Assembly Subtotal |  |  |  | ₹ 29,000.00 |  |
| GRAND TOTAL |  |  |  | ₹ 1,65,580.30 |  |

| Notes & Terms • Prices are indicative and subject to supplier confirmation. • Amounts of Components are inclusive of GST unless stated otherwise. • All amounts in Indian Rupees (₹). | ________________________ Sakthivelan R Authorized Signature |
| --- | --- |